SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: 	COMPUTER ARCHITECTURE COURSE CODE: 	CSA12

COURSE OUTCOME COVERED

CO1: Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) Assessment Tool Weightage: 40%

QUESTIONS:5 	 	 	 	 	 	Total Marks:25

ANNEXURE A

APPLICATION BASED QUESTIONS

Code of Conduct:

I ATHISAYA U (192571001)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ANNEXURE A

1. A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

A smart city surveillance system continuously receives high-resolution video from several cameras. This produces a large amount of data that must be transferred from the cameras to memory and then processed by the CPU for threat detection. The delay experienced during peak hours is mainly caused by a bottleneck in the interaction between the CPU, memory and I/O devices.

Interaction between functional units

The cameras act as high-speed I/O devices. Their video frames are first placed in buffers and transferred to main memory. The CPU then fetches the required data and instructions from memory, processes the frames and generates alerts.

Performance is reduced when:

- Multiple cameras send data simultaneously.

- The CPU spends time controlling I/O transfers instead of analysing video.

- Main memory cannot supply video frames at the required speed.

- Cache misses force the CPU to wait for slower main memory.

- CPU, memory and I/O devices compete for the same communication bus.

- Large frames repeatedly move between I/O devices, memory and the processor.

Thus, even a fast CPU may remain idle when it is waiting for video data.

Single-bus structure

In a single-bus system, the CPU, memory and all I/O devices share one common bus.

During peak hours, only one data transfer can take place at a time. For example, when a camera transfers a video frame to memory, the CPU may have to wait before accessing the same bus. This creates bus contention and increases response time.

Advantages:

- Simple design.

- Lower cost.

- Easy to control.

Limitations:

- Only one transfer occurs at a time.

- High traffic creates a bottleneck.

- Poor performance for continuous video processing.

Multi-bus structure

A multi-bus architecture provides separate or multiple paths for communication. For example, one bus may handle CPU memory communication while another handles I/O transfers.

This allows camera data to be transferred to memory while the CPU simultaneously executes instructions or processes a previous video frame.

Advantages:

- Parallel data transfers are possible.

- Bus contention is reduced.

- Memory and I/O operations can overlap.

- Better scalability when more cameras are added.

- Faster threat detection and response.

Improvements to instruction execution

System performance can be enhanced through:

- Instruction pipelining: Fetch, decode and execution of different instructions are overlapped.

- Cache memory: Frequently used instructions and video data are stored close to the CPU.

- Direct Memory Access: DMA transfers video data directly from cameras to memory without continuous CPU involvement.

- Prefetching: Required instructions and data are loaded before the CPU requests them.

- Parallel processing: Multiple CPU cores or GPUs can process video feeds simultaneously.

- Interrupt-based I/O: The CPU performs useful work until an I/O device requires attention.

- Larger buffers: Temporary storage prevents data loss during sudden increases in traffic.

Conclusion

The delay occurs because large video transfers overload the shared communication path and force the CPU to wait for memory and I/O operations. Replacing a single-bus design with a multi-bus architecture, along with DMA, caching, pipelining and parallel processing, significantly improves throughput and enables faster real-time threat detection

2.A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode– execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

The performance of a stock trading application depends on how quickly the processor completes the fetch–decode–execute cycle. During high load, millions of instructions must process market updates, verify orders, calculate prices and complete transactions.

The CPU execution time can be expressed as:

CPU Execution Time =  Instruction Count*CPI*Clock Cycle Time

Where:

- Instruction Count is the total number of executed instructions.

- CPI is the average number of clock cycles required per instruction.

- Clock Cycle Time is the duration of one processor clock cycle.

Effect of the fetch–decode–execute cycle

1. Fetch

The CPU fetches the instruction from memory using the address in the program counter. If the instruction is not available in cache, the CPU must access slower main memory, increasing CPI.

2. Decode

The control unit decodes the instruction and identifies the required operation, operands and addressing mode. Complex instructions may require additional decoding time.

3. Execute

The ALU performs calculations, memory is accessed, or a branch is executed. Load and store instructions may cause memory delays, while branch instructions may disturb the pipeline.

4. Store result

The result is written into a register or memory. Writing to memory may add further delay.

Reasons for high CPI

The major causes are:

- Frequent cache misses.

- Excessive load and store instructions.

- Slow main-memory access.

- Frequent conditional branches.

- Branch misprediction.

- Pipeline stalls.

- Data dependencies between instructions.

- Competition between processing cores for shared memory.

A wrongly predicted branch may cause already-fetched instructions to be discarded. The pipeline must then be refilled, wasting several clock cycles.

Architectural improvements

Cache memory

Larger and faster L1, L2 and L3 caches reduce the number of main-memory accesses. Frequently accessed stock prices and transaction records can remain in cache.

Branch prediction

A dynamic branch predictor estimates whether a branch will be taken. Accurate prediction prevents pipeline flushing and reduces branch penalties.

Instruction pipelining

Pipelining overlaps fetch, decode and execution. After the pipeline is filled, the processor can ideally complete approximately one instruction per clock cycle.

Out-of-order execution

The CPU can execute independent instructions while another instruction waits for data from memory. This reduces idle processor cycles.

Register usage

Frequently used values should be stored in CPU registers instead of repeatedly accessing memory.

Prefetching

Data and instructions likely to be required next can be brought into the cache in advance.

Multicore processing

Independent tasks such as market-data reception, risk analysis and order processing can be distributed across multiple processor cores.

Software optimisation

The number of unnecessary instructions, memory accesses and branches should be reduced. Efficient data structures and branch-free algorithms can further lower CPI.

Conclusion

Transaction delays occur because memory accesses and branch instructions introduce additional clock cycles into the fetch–decode–execute cycle. Cache improvement, pipelining, branch prediction, out-of-order execution, prefetching and multicore processing reduce CPI and allow transactions to be processed with lower latency.

3.An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

In an industrial temperature controller, the 8085 receives temperature data through an input port, compares it with a required limit and controls actuators such as fans, heaters or alarms. Incorrect use of addressing modes can cause the processor to read the wrong data or access the wrong memory location.

Addressing modes of the 8085

Immediate addressing

The data is included directly in the instruction.

MVI A, 32H

Here, 32H is the actual data loaded into the accumulator.

It is useful for loading fixed temperature limits but may cause errors if the programmer treats the value as a memory address.

Register addressing

The operand is stored in a processor register.

MOV A, B

The contents of register B are copied into the accumulator.

It provides fast access but incorrect results occur if the register has not been properly initialised.

Direct addressing

The complete memory address is written in the instruction.

LDA 2500H

The accumulator receives the data stored at memory location 2500H.

An incorrect address may cause the program to read unrelated or outdated sensor data.

Register-indirect addressing

The memory address is stored in a register pair, normally HL.

LXI H, 2500H

MOV A, M

M represents the memory location addressed by the HL pair.

If HL contains the wrong address, the processor accesses the wrong memory location.

Implicit addressing

The operand is automatically understood from the instruction.

CMA

The accumulator is implicitly used. No separate operand is specified.

Possible causes of incorrect readings

- Confusing immediate data with a memory address.

- Loading the wrong address into the HL register pair.

- Using an incorrect I/O port address.

- Accessing a memory location before sensor data is updated.

- Failing to initialise registers.

- Sensor data being overwritten by another operation.

- Reading data during an incomplete sensor conversion.

- Incorrect masking of status or control bits.

- Incorrect comparison with the threshold value.

Reliable use of the 8085 instruction set

The input can be read using the IN instruction:

IN 80H

The value can be stored safely in memory:

STA 2500H

A threshold can be loaded and compared:

MVI B, 32H

CMP B

Conditional jump instructions can then control the actuator:

JC TEMPERATURE_LOW

JZ TEMPERATURE_NORMAL

JNC TEMPERATURE_HIGH

The required actuator can be activated using:

OUT 81H

Accuracy can be improved by:

- Initialising all registers and stack values.

- Using correct port and memory addresses.

- Reading the sensor more than once and averaging the readings.

- Checking sensor-ready status before accepting data.

- Storing readings in dedicated memory locations.

- Using interrupts for timely sensor sampling.

- Preserving important register values with PUSH and POP.

- Testing boundary values such as minimum, normal and maximum temperatures.

Conclusion

Addressing modes determine whether an instruction accesses immediate data, a register, a memory location or an I/O port. Accurate selection of addressing modes, proper register initialisation and correct use of IN, OUT, CMP and conditional jump instructions ensure reliable temperature measurement and actuator control.

4.A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

The 8086 microprocessor has a 20-bit address bus and can access up to 1 MB of memory. Since its registers are only 16 bits wide, it uses segmented memory. The physical address is calculated using:

Physical Address = Segment Address × 10H + Offset Address

For example, if DS = 2000H and offset = 1500H:

Physical Address = 2000H × 10H + 1500H = 21500H

The main memory segments in 8086 are:

1.CodeSegment:
The Code Segment register, CS, stores the starting address of the program instructions. The Instruction Pointer, IP, stores the offset of the next instruction. The processor fetches instructions using the CS:IP combination.

2.DataSegment:
The Data Segment register, DS, points to the memory area that stores variables, arrays and other program data. Most data-access instructions use DS by default.

3.StackSegment:
The Stack Segment register, SS, points to the stack area, while the Stack Pointer, SP, indicates the top of the stack. The stack is used during PUSH, POP, CALL, RET and interrupt operations.

4.ExtraSegment:
The Extra Segment register, ES, provides an additional data segment and is mainly used in string operations.

Incorrect data access can occur when DS is not properly initialised, when an incorrect segment address is loaded, when the offset exceeds the intended memory area or when segment registers are changed without restoring them. For example, if DS points to the wrong segment, the instruction:

MOV AX, [1200H]

will read data from an incorrect physical address.

Stack overflow or stack corruption may occur because of excessive PUSH operations, missing POP instructions, deep recursive calls, incorrect SS and SP initialisation or improper use of CALL and RET instructions. Since the stack grows towards lower memory addresses, it may overwrite program data when enough stack space is not allocated.

A proper segment initialisation can be written as:

MOV AX, DATA_SEGMENT
MOV DS, AX
MOV AX, STACK_SEGMENT
MOV SS, AX
MOV SP, STACK_TOP

To ensure correct execution:

- CS, DS, SS and ES must be properly initialised.

- Separate memory areas must be allocated for code, data and stack.

- Every PUSH instruction must have a matching POP instruction.

- Every CALL instruction must have the correct RET instruction.

- Segment registers must be saved and restored during procedures.

- Offsets must remain within the valid segment area.

- Correct addressing modes must be used.

The 8086 supports direct, register indirect, based, indexed and based-indexed addressing modes. Instructions using BP normally access the stack segment, whereas BX, SI and DI usually access the data segment.

Therefore, proper management of segment registers, stack operations, offsets and addressing modes is essential to prevent incorrect data access, stack overflow and program failure.

5.A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

Data communication systems commonly represent packet information in hexadecimal because hexadecimal values are shorter and easier to read than binary values. Each hexadecimal digit represents exactly four binary bits.

For example:

3AH = 0011 1010B

Here, hexadecimal 3 is equal to binary 0011 and hexadecimal A is equal to binary 1010.

Similarly:

F0H = 1111 0000B

Packet headers may contain source and destination addresses, packet length, protocol identifiers, control flags, error-checking values and port numbers. These fields may be displayed in hexadecimal for debugging, but the CPU processes them as binary values. Therefore, correct conversion is very important.

For example:

0CH = 0000 1100B

If it is incorrectly converted as 1100 0000B, the processor will check the wrong control bits. This may cause the packet to be rejected, routed incorrectly or interpreted as a different command.

Errors in hexadecimal-to-binary conversion can affect instruction execution in several ways.

1. Incorrect Opcode:

A wrong hexadecimal digit may represent a completely different machine instruction.

2. Wrong Memory Address:

An incorrect conversion may cause the CPU to access the wrong memory location.

3. Incorrect Operand:

The processor may perform calculations using the wrong numerical value.

4. Wrong Bit Mask:

The program may test, set or clear an incorrect control bit.

5. Incorrect Packet Length:

The CPU may read too many or too few bytes, causing incomplete packets, buffer overflow or data corruption.

6. Signed Number Error:

Incorrect handling of the most significant bit may cause a negative number to be interpreted as positive or vice versa.

Conversion errors can be reduced by:

- Converting every hexadecimal digit into exactly four binary bits.

- Using standard conversion functions instead of manual conversion.

- Validating packet length and field values.

- Applying correct masks and shift operations.

- Using checksums or CRC for error detection.

- Testing the system with known hexadecimal and binary values.

- Displaying both hexadecimal and binary values during debugging.

Efficient CPU design also improves real-time packet processing. Pipelining allows different instruction stages to overlap. Cache memory stores frequently used packet data close to the CPU. Fast bitwise instructions help in masking and shifting operations. Parallel processing allows several packet operations to be performed at the same time. Hardware support for checksum and CRC calculation can also reduce processing time.

Benchmarking helps measure and identify system problems. Important performance measures include:

- Packet-processing time.

- Number of packets processed per second.

- Average CPI.

- Cache-miss rate.

- Branch-misprediction rate.

- CPU utilisation.

- Memory bandwidth.

- Error-detection rate.

Known packet values can be processed and compared with expected outputs. If an incorrect conversion or excessive delay occurs, benchmarking helps identify the stage responsible for the problem.

Therefore, accurate hexadecimal-to-binary conversion is essential because conversion errors can change instructions, addresses, operands and packet-control information. Reliable conversion methods, efficient CPU architecture and benchmarking help improve accuracy and real-time system performance.

MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| ProblemSolving Approach | 20 | Logical, wellstructured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, wellorganized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |